METHANE VISUALIZATION ANALYSIS NEW

Great — with 915 MHz approved, methane sensor data, and up to 5 lbs of payload capacity, you’re in excellent shape to build a robust long-range wireless data-visualization link that avoids Wi-Fi entirely.

And because you’re only transmitting low-bandwidth methane readings, not video or imagery, we can choose extremely reliable, field-proven telemetry systems used in drones, agriculture, and research UAVs.

Below is your complete recommended architecture, a shopping list, and a setup plan tailored to your exact needs.

# ✅ Recommended System: 915 MHz Digital Telemetry Link (SiK-type or RFD900)

This gives you:

- Reliable hundreds of feet to miles range

- Digital data, not analog video

- Non-Wi-Fi interference-resistant link

- Real-time visualization on ground

- Very small bandwidth use (perfect for methane ppm, %LEL, etc.)

# 🟦 Option A — SiK / 3DR-Style 915 MHz Telemetry Radios (Best Value)

These are the standard for drones and environmental monitoring. They create a transparent serial link (like a wireless USB cable).

### Why they work for your use case

- Designed for drones & research payloads

- Extremely reliable at 1000+ ft line-of-sight

- Very low power & weight

- Enough bandwidth for methane sensor telemetry

- Easy to connect to Raspberry Pi (UART pins)

### Physical setup

- One radio goes on the drone with your Raspberry Pi

- The second radio plugs into your laptop via USB

- Your Pi sends serial data → instant visualization on your laptop dashboard

# 🟦 Option B — RFD900x / RFD900+ (Premium, Most Reliable)

If you want bulletproof reliability or plan to take this into field deployments, RFD radios are industry standards in UAVs, agriculture, methane leak-mapping, and research.

They are more expensive but offer:

- Extremely robust link

- AES encryption

- 5–40+ km range (overkill but ultra-stable at short distances)

- Frequency hopping for noise immunity

# ✔ You have MORE than enough payload capacity

These radios weigh only 30–50 grams.
Even with antennas, mounting hardware, and wiring, you're using < 0.25 lbs of your additional 5 lbs capacity.

So weight is not an issue for you at all.

# 📡 System Diagram (Simple + Reliable)

## Drone

- Raspberry Pi

- Your custom methane sensor payload

- SiK 915 MHz Telemetry Radio (Air Module)

- 915 MHz antenna

## Ground Station

- SiK 915 MHz Telemetry Radio (Ground Module)

- USB → Laptop

- Python dashboard (I can build this for you)

# 📦 Shopping List (Specific Recommendations)

Here are proven, drone-compatible radios you can purchase today.

## ✔ 1. Holybro 500mW 915 MHz SiK Telemetry Radio Set

- Lightweight

- Plug-and-play

- Highly reliable

- USB ground unit + air radio included

## ✔ 2. RFD900x or RFD900ux

- Enterprise-grade

- Long range

- Uses frequency hopping for stable link

- Best choice if this could expand into full research flights

## ✔ 3. Accessories

- 915 MHz whip antennas (included with most sets)

- UART cables for Raspberry Pi

- USB cable for ground station radio

- Optional: 3S or 5V BEC power supply if needed

# 🔧 Raspberry Pi Wiring (Simple)

Your Raspberry Pi uses UART to communicate with the radio.

### Pins:

- Pi TX (GPIO14) → Radio RX

- Pi RX (GPIO15) → Radio TX

- Pi 5V → Radio 5V input

- Pi GND → Radio GND

# 🧪 Software Architecture (Very Effective)

Your Pi is already writing methane data to the SD card — perfect.

To visualize it live, you simply:

- Open a serial port on the Pi:
serial = UART("/dev/ttyAMA0", 57600)

- Send each methane reading:
serial.write(f"{timestamp},{methane_ppm}\n")

- On the laptop, read the incoming serial stream and plot it visually
(I can generate a ready-to-run Python dashboard for you)

# 📈 What You Can Visualize in Real Time

Because the link is digital, you can stream:

- Methane concentration (ppm, %LEL, raw ADC)

- GPS

- Temperature / humidity

- Battery voltage

- Any text-based telemetry

- Alerts (e.g., methane spike detected)

All while the drone is moving, hundreds of feet away.

# 🔒 Why This Solves All the Problems Wi-Fi Can’t

You mentioned several reasons Wi-Fi won’t work.
Telemetry radios fix all of them:

| Problem with Wi-Fi | Telemetry Radio Solution |
| --- | --- |
| Limited range | Works 1000 ft+ easily |
| High interference | Uses sub-GHz with FHSS |
| Security issues | Encrypted modes available |
| Unreliable on moving drones | Designed for UAV telemetry |
| Needs high bandwidth | Sensor data is small, perfect fit |

This is the standard solution used in environmental and agricultural drone research.

# 👍 Next Steps (I can prepare these for you)

If you'd like, I can now create:

### ✔ A full wiring diagram

### ✔ A complete Raspberry Pi script

### ✔ A ground-station visualization dashboard

### ✔ A shopping cart with all parts & links

### ✔ A step-by-step setup guide

To finish tailoring the system, one last useful detail:

### Do you want the methane sensor data streaming live, AND saving to the SD card, or just streaming live?

Once you answer that, I’ll build your entire integration package.

Great — with 915 MHz approved, methane sensor data, and up to 5 lbs of payload capacity, you’re in excellent shape to build a robust long-range wireless data-visualization link that avoids Wi-Fi entirely.

And because you’re only transmitting low-bandwidth methane readings, not video or imagery, we can choose extremely reliable, field-proven telemetry systems used in drones, agriculture, and research UAVs.

Below is your complete recommended architecture, a shopping list, and a setup plan tailored to your exact needs.

# ✅ Recommended System: 915 MHz Digital Telemetry Link (SiK-type or RFD900)

This gives you:

- Reliable hundreds of feet to miles range

- Digital data, not analog video

- Non-Wi-Fi interference-resistant link

- Real-time visualization on ground

- Very small bandwidth use (perfect for methane ppm, %LEL, etc.)

# 🟦 Option A — SiK / 3DR-Style 915 MHz Telemetry Radios (Best Value)

These are the standard for drones and environmental monitoring. They create a transparent serial link (like a wireless USB cable).

### Why they work for your use case

- Designed for drones & research payloads

- Extremely reliable at 1000+ ft line-of-sight

- Very low power & weight

- Enough bandwidth for methane sensor telemetry

- Easy to connect to Raspberry Pi (UART pins)

### Physical setup

- One radio goes on the drone with your Raspberry Pi

- The second radio plugs into your laptop via USB

- Your Pi sends serial data → instant visualization on your laptop dashboard

# 🟦 Option B — RFD900x / RFD900+ (Premium, Most Reliable)

If you want bulletproof reliability or plan to take this into field deployments, RFD radios are industry standards in UAVs, agriculture, methane leak-mapping, and research.

They are more expensive but offer:

- Extremely robust link

- AES encryption

- 5–40+ km range (overkill but ultra-stable at short distances)

- Frequency hopping for noise immunity

# ✔ You have MORE than enough payload capacity

These radios weigh only 30–50 grams.
Even with antennas, mounting hardware, and wiring, you're using < 0.25 lbs of your additional 5 lbs capacity.

So weight is not an issue for you at all.

# 📡 System Diagram (Simple + Reliable)

## Drone

- Raspberry Pi

- Your custom methane sensor payload

- SiK 915 MHz Telemetry Radio (Air Module)

- 915 MHz antenna

## Ground Station

- SiK 915 MHz Telemetry Radio (Ground Module)

- USB → Laptop

- Python dashboard (I can build this for you)

# 📦 Shopping List (Specific Recommendations)

Here are proven, drone-compatible radios you can purchase today.

## ✔ 1. Holybro 500mW 915 MHz SiK Telemetry Radio Set

- Lightweight

- Plug-and-play

- Highly reliable

- USB ground unit + air radio included

## ✔ 2. RFD900x or RFD900ux

- Enterprise-grade

- Long range

- Uses frequency hopping for stable link

- Best choice if this could expand into full research flights

## ✔ 3. Accessories

- 915 MHz whip antennas (included with most sets)

- UART cables for Raspberry Pi

- USB cable for ground station radio

- Optional: 3S or 5V BEC power supply if needed

# 🔧 Raspberry Pi Wiring (Simple)

Your Raspberry Pi uses UART to communicate with the radio.

### Pins:

- Pi TX (GPIO14) → Radio RX

- Pi RX (GPIO15) → Radio TX

- Pi 5V → Radio 5V input

- Pi GND → Radio GND

# 🧪 Software Architecture (Very Effective)

Your Pi is already writing methane data to the SD card — perfect.

To visualize it live, you simply:

- Open a serial port on the Pi:
serial = UART("/dev/ttyAMA0", 57600)

- Send each methane reading:
serial.write(f"{timestamp},{methane_ppm}\n")

- On the laptop, read the incoming serial stream and plot it visually
(I can generate a ready-to-run Python dashboard for you)

# 📈 What You Can Visualize in Real Time

Because the link is digital, you can stream:

- Methane concentration (ppm, %LEL, raw ADC)

- GPS

- Temperature / humidity

- Battery voltage

- Any text-based telemetry

- Alerts (e.g., methane spike detected)

All while the drone is moving, hundreds of feet away.

# 🔒 Why This Solves All the Problems Wi-Fi Can’t

You mentioned several reasons Wi-Fi won’t work.
Telemetry radios fix all of them:

| Problem with Wi-Fi | Telemetry Radio Solution |
| --- | --- |
| Limited range | Works 1000 ft+ easily |
| High interference | Uses sub-GHz with FHSS |
| Security issues | Encrypted modes available |
| Unreliable on moving drones | Designed for UAV telemetry |
| Needs high bandwidth | Sensor data is small, perfect fit |

This is the standard solution used in environmental and agricultural drone research.

# 👍 Next Steps (I can prepare these for you)

If you'd like, I can now create:

### ✔ A full wiring diagram

### ✔ A complete Raspberry Pi script

### ✔ A ground-station visualization dashboard

### ✔ A shopping cart with all parts & links

### ✔ A step-by-step setup guide

To finish tailoring the system, one last useful detail:

### Do you want the methane sensor data streaming live, AND saving to the SD card, or just streaming live?

Once you answer that, I’ll build your entire integration package.

Perfect—that keeps things simple now and leaves room to grow later.

Below is a complete, NDAA-compliant plan for a live scrolling strip-chart from your methane payload while also logging to the Pi’s SD card. It uses a 915 MHz digital telemetry link (not Wi-Fi) that’s robust for a moving drone over hundreds of feet.

## System overview

Air (on the drone)

- Raspberry Pi (your current payload computer)

- Methane sensor interface (existing)

- 915 MHz telemetry radio (RFD900x/ux or Microhard pDDL; NDAA-compliant)

- Lightweight 915 MHz antenna

- UART cable Pi ⇄ radio

- Pi software that (a) logs to SD, and (b) streams light-weight telemetry lines over UART

Ground

- Matching 915 MHz telemetry radio (USB model)

- Laptop (Windows/macOS/Linux)

- Simple Python viewer that opens the USB serial port and plots a live scrolling chart of methane vs. time, while optionally saving a mirrored CSV locally

## Recommended NDAA-compliant hardware

Weight is trivial (tens of grams), well under your 5 lb margin.

Option A (most robust, research-grade):

- RFD900x or RFD900ux telemetry radio pair (915 MHz)

  - Pros: FHSS, long-range, excellent interference rejection, encryption options, widely used in UAS research and federal projects.

  - What to order: “RFD900x/ux 915 MHz set” (includes 1× air unit, 1× ground unit, antennas, USB interface for ground).

Option B (cost-sensitive, still NDAA-safe):

- Microhard pDDL (915 MHz) pair

  - Pros: Compact, industrial-grade Ethernet/serial bridge options.

  - Note: Configuration is a bit more involved than RFD, but rock-solid.

Accessories

- Pi UART harness (JST-GHR or DF13 depending on radio)

- 915 MHz dipole/quarter-wave antennas (usually included)

- Optional: 5 V BEC (if you prefer not to power the air radio from the Pi’s 5 V rail)

If you’d like, I can assemble a purchase list with exact part numbers after you choose RFD900x/ux vs Microhard.

## Wiring (air side)

Raspberry Pi ⇄ Radio (UART)

- Pi GPIO14 (TXD) → Radio RX

- Pi GPIO15 (RXD) → Radio TX

- Pi 5 V → Radio 5 V (or use external 5 V BEC)

- Pi GND → Radio GND

Antenna placement tips

- Mount the 915 MHz antenna away from high-current ESC leads and the methane sensor head.

- Keep at least ~5–10 cm separation from other antennas; use vertical orientation for best omnidirectional coverage.

## Radio configuration (quick start)

- Set both radios to 915 MHz region (US) and matching air data rate (e.g., 57,600 or 115,200 bps).

- Keep default FHSS settings; select a Network ID unique to your system.

- For short-range reliability, 20–100 mW TX power is often plenty; increase only if you observe dropouts.

- (Optional) Enable AES encryption if your compliance rules or data sensitivity require it.

## Raspberry Pi software (log + live stream)

Below is a minimal Python pattern. It:

- Reads a methane value from your sensor function read_methane().

- Appends to /home/pi/methane_log.csv.

- Streams a CSV line over UART to the ground radio.

- Uses a ring buffer on the Pi’s filesystem implicitly by line-by-line writes (safe for power cuts if you flush()).

Replace read_methane() with your actual acquisition call.

# file: airtx.py

import time, os, serial, csv

LOGPATH = "/home/pi/methanelog.csv"

SERIALPORT = "/dev/ttyAMA0"   # or /dev/serial0 depending on your Pi image

BAUD = 57600                   # match your radios

def readmethane():

# TODO: replace with your sensor read, return float (ppm or %LEL)

# e.g., value = mysensor.getppm()

return 1.234

def ensurelogexists():

newfile = not os.path.exists(LOGPATH)

f = open(LOGPATH, "a", newline="")

writer = csv.writer(f)

if newfile:

writer.writerow(["timestamps", "methanevalue"])

f.flush()

return f, writer

def main():

ser = serial.Serial(SERIALPORT, BAUD, timeout=1)

f, writer = ensurelogexists()

try:

while True:

ts = time.time()

value = read_methane()

# 1) Append to SD card

writer.writerow([f"{ts:.3f}", f"{value:.6f}"])

f.flush()  # ensure near-real-time durability

# 2) Stream to ground as CSV line

line = f"{ts:.3f},{value:.6f}\n"

ser.write(line.encode("utf-8"))

ser.flush()

time.sleep(0.5)  # 2 Hz update; adjust as desired

except KeyboardInterrupt:

pass

finally:

f.close()

ser.close()

if name == "main":

main()

Enable the UART on the Pi

- sudo raspi-config → Interface Options → Enable Serial (disable login shell over serial; enable hardware serial).

- Reboot.

## Ground station viewer: live scrolling strip-chart

This simple script:

- Opens the ground radio’s USB COM port.

- Parses the incoming timestamp,value lines.

- Displays a live, auto-scrolling plot (last N minutes).

- Optionally mirrors data into a local CSV for post-flight.

# file: groundviewer.py

import sys, csv, time, serial, collections

import matplotlib.pyplot as plt

import matplotlib.animation as animation

PORT = "COM7"         # Windows example. On macOS: "/dev/tty.usbserial-XXXX"

BAUD = 57600

MIRRORCSV = "groundmethanelog.csv"

WINDOWSECONDS = 300   # display last 5 minutes

def main():

ser = serial.Serial(PORT, BAUD, timeout=1)

mirror = open(MIRRORCSV, "a", newline="")

mwriter = csv.writer(mirror)

# header if file is empty

if mirror.tell() == 0:

mwriter.writerow(["timestamps", "methanevalue"])

mirror.flush()

data = collections.deque(maxlen=2000)  # stores (t, v)

fig, ax = plt.subplots(figsize=(8,4))

line, = ax.plot([], [], lw=2)

ax.settitle("Live Methane (strip-chart)")

ax.setxlabel("Time (s, relative)")

ax.setylabel("Methane (units)")

t0 = None

def update(frame):

nonlocal t0

# read all available lines

while ser.inwaiting:

raw = ser.readline().decode("utf-8", errors="ignore").strip()

if not raw:

continue

try:

tsstr, valstr = raw.split(",")

ts = float(tsstr)

val = float(valstr)

except Exception:

continue

if t0 is None:

t0 = ts

data.append((ts - t0, val))

mwriter.writerow([f"{ts:.3f}", f"{val:.6f}"])

mirror.flush()

if not data:

return line,

# Keep only last WINDOWSECONDS

tmax = data[-1][0]

while data and (tmax - data[0][0]) > WINDOWSECONDS:

data.popleft()

xs = [p[0] for p in data]

ys = [p[1] for p in data]

line.setdata(xs, ys)

ax.setxlim(max(0, tmax - WINDOWSECONDS), tmax + 2)

# Auto-scale Y with margin

ymin = min(ys); ymax = max(ys)

pad = max(1e-6, (ymax - ymin) * 0.1)

ax.setylim(ymin - pad, ymax + pad)

return line,

ani = animation.FuncAnimation(fig, update, interval=200)

plt.tightlayout()

plt.show()

mirror.close()

ser.close()

if name == "main":

if len(sys.argv) > 1:

PORT = sys.argv[1]  # allow: python ground_viewer.py /dev/ttyUSB0

main()

Run:

- Windows: python ground_viewer.py COM7

- macOS/Linux: python ground_viewer.py /dev/tty.usbserial-XXXX or /dev/ttyUSB0

## Operational checklist

- On the bench

  - Power Pi and air radio; power ground radio via USB.

  - Verify serial link by sending a test line from Pi and seeing it appear in a serial terminal on the laptop (e.g., screen/PuTTY).

  - Tune BAUD, TX power low (20–100 mW), unique NetID.

- Flight readiness

  - Confirm antenna mounts are secure and vertical.

  - Confirm methane sensor is shielded from RF as needed (most are fine at 915 MHz; if sensitive, add a small metal ground plate between radio and sensor).

- During flight

  - Start air_tx.py on the Pi (e.g., systemd service).

  - Start ground_viewer.py on your laptop; confirm scrolling chart.

- After flight

  - Retrieve /home/pi/methane_log.csv from the Pi as your authoritative dataset.

  - The ground mirror CSV is a backup and for quick analysis.

## Future-ready upgrades (when you’re ready)

- A) Add GPS on the Pi and render methane vs. position (map overlay).

- B) Add thresholds and audible alarms on the ground viewer.

- C) Switch to a Plotly Dash UI for richer UI + exportable PNGs.

- D) Use encryption and signed packets for compliance/security.

- E) Add forward-error correction/packet CRC with a tiny line protocol (CSV → framed messages).

I can generate any of these when you’re ready; the radio link and wiring stay the same.

## Want me to package this for you?

If you’d like, I can create a zip with ready-to-run code plus a PDF quick-start (and even a systemd service file for autostart on boot). Say the word and I’ll produce the files and share them here for download.

### Quick confirm

Do you prefer RFD900x/ux or Microhard pDDL for the radios? Once you choose, I’ll give you a specific bill of materials with part numbers and a one-page wiring diagram tailored to that radio.

Awesome—going with RFD900x/ux keeps you NDAA-compliant and rock-solid for a moving drone telemetry link. I’ve packaged ready-to-run code, plus a 1-page quick-start and a 1-page wiring diagram, and I’ll give you the BOM and setup notes below.

## ⬇️ Downloads (code + docs)

- Complete ZIP package (code + PDFs):\ rfd900_link_kit.zip\ What’s inside:

  - air_tx.py – Pi-side transmitter (logs to SD and streams over UART)

  - ground_viewer.py – Laptop live strip-chart viewer (scrolling)

  - services/air_tx.service – systemd unit (optional auto-start)

  - QuickStart.pdf – 1-page setup checklist

  - WiringDiagram_RFD900x_ux.pdf – 1-page Pi⇄RFD900x/ux UART wiring

The code sends CSV lines like timestamp_s,methane_value at 2 Hz by default, which is well within the RFD900 data link capability (multi-hundreds of kbps air rates and 40 km+ LoS range under proper antennas). [us-prod.as…rosoft.com], [files.rfde…ign.com.au]

## 🧰 NDAA-Compliant Bill of Materials (tailored for RFD900x/ux)

Why RFD900x/ux: RFDesign explicitly lists the RFD900x as NDAA compliant, designed/tested in Australia, with US-compliant geographic variants and FCC Part 15.247 alignment. The ux series is the smaller, pin-compatible variant (u.FL connectors) and is also used in US projects, with “-US” SKUs pre-set for the US band. [Microsoft…- RFDesign]

Tip: For the air side, most teams choose the compact RFD900ux-US, and for the ground side a full-size RFD900x-US with USB interface—functionally interoperable. Specs include diversity antennas, up to +30 dBm TX, and air data rates up to 224–500 kbps depending on firmware/datasheet version. [rfdesign.com.au], [us-prod.as…rosoft.com]

### Recommended (mixed) configuration

- Air (on drone):

  - RFD900ux-US modem (compact; u.FL antenna ports; 915 MHz US variant) — SKU often shown as MODEM-RFD900ux-US. [RFD900ux D…- RFDesign], [rfdesign.com.au]

- Ground (laptop):

  - RFD900x-US modem (full-size; RP-SMA; 915 MHz US variant) — MPN MODEM-RFD900x-US. You can buy the bare modem and add an FTDI 3.3 V USB cable to make it a USB ground unit. [nwblue.com]

### Accessories

- Antenna(s):

  - For RFD900x-US: 900 MHz RP-SMA monopole/dipole (often included in “air” or “bundle” kits; some sellers do not include antennas). [irlock.com], [3dxr.co.uk]

  - For RFD900ux-US: two u.FL → RP-SMA pigtails + 900 MHz dipoles; many ux-US bundles include multiple antennas and cables. [RFD900ux D…- RFDesign]

- UART Cable (Pi → RFD): 6-pin JST-GH cable for RFD900x/ux (e.g., “RFD to 6-pin JST-GH” 300 mm). [irlock.com]

- USB for ground modem: FTDI 3.3 V (if your kit does not include a USB interface). Some bundles include this; others (e.g., some RMRC listings) do not. [3dxr.co.uk]

- (Optional) 5 V BEC: If you prefer not to power the air modem from the Pi’s 5 V rail.

### Alternate “bundle” route (fastest to buy/deploy)

- RFD900ux-US Telemetry Bundle (pair of ux-US modems, antennas, pigtails, FTDI cable; marked as Blue UAS / NDAA compliant and manufactured in allied nations). This can cover both air and ground in one purchase. [RFD900ux D…- RFDesign]

## 🔌 Wiring (matches the included diagram)

Raspberry Pi → RFD900x/ux (Air Unit)

- Pi GPIO14 (TXD, pin 8) → RFD RX (3.3 V TTL)

- Pi GPIO15 (RXD, pin 10) ← RFD TX (3.3 V TTL)

- Pi 5 V (pin 2 or 4) → RFD +5 V

- Pi GND (e.g., pin 6) → RFD GND

Keep the antenna(s) vertical and separated from high-current ESC wires. The RFD900x/ux supports dual-antenna diversity; use the two ports with matching 915 MHz antennas (RP-SMA on x, u.FL on ux). [us-prod.as…rosoft.com], [rfdesign.com.au]

## ⚙️ Radio configuration (quick checklist)

- Region/variant: Use the US variant (-US) locked to 902-928 MHz; that’s the correct ISM band for the US. [Microsoft…- RFDesign]

- Baud & link rate: Start with 57600 UART baud and 64 kbps air rate (default/common), then tune as needed. [nwblue.com]

- TX power: For “hundreds of feet”, 20–100 mW is usually sufficient; increase only if you see dropouts. (The radios can go up to +30 dBm, but that’s overkill at short range.) [us-prod.as…rosoft.com]

- NetID: Choose a unique Network ID on both radios to avoid crosstalk. (Managed via AT commands/APM Planner/RFD Tools.) [nwblue.com]

- Compliance: RFD900x is designed to comply with FCC Part 15.247, and RFDesign marks the product NDAA compliant; US-locked SKUs simplify adherence. [Microsoft…- RFDesign]

## 💻 How to run (summary)

- Air (Pi):

# enable UART (raspi-config → Serial: login shell No, hardware serial Yes)

python3 /home/pi/rfd900linkkit/airtx.py

# optional: systemd auto-start

sudo cp services/airtx.service /etc/systemd/system/

sudo systemctl enable --now air_tx.service

- Ground (Laptop):

# Windows example

python3 groundviewer.py COM7

# Linux example

python3 groundviewer.py /dev/ttyUSB0

# macOS example

python3 ground_viewer.py /dev/tty.usbserial-XXXX

The strip-chart auto-scrolls (last 5 min by default). A mirrored CSV is saved locally as a backup.

## 📎 Why this meets your requirements

- NDAA-compliant radios (RFD900x/ux) with US variants and posted NDAA statement by the manufacturer (Australia-based, US-compliant geographic SKUs). [Microsoft…- RFDesign]

- Long-range digital telemetry appropriate for a moving drone, not Wi-Fi, with FHSS and robust link budgets (tens of km in spec sheets—so hundreds of feet are trivial). [us-prod.as…rosoft.com], [files.rfde…ign.com.au]

- Low-bandwidth sensor streaming + SD logging via simple UART CSV—exactly what these modems are built for. [us-prod.as…rosoft.com]

## ✅ Next steps I can do for you

- If you want me to pre-configure RFD parameter files (e.g., NetID, channel map, TX power) or add GPS mapping in the viewer later, I can extend the package.

- If you prefer a single-vendor bundle to simplify procurement (e.g., RFD900ux-US telemetry bundle with antennas and FTDI), I can format a cart for you and confirm stock/lead times. [RFD900ux D…- RFDesign]

Before you head to procurement, do you want me to:

- Lock the recommendation to RFD900ux-US (air) + RFD900x-US (ground),\ or 2) Make both ends RFD900x-US,\ or 3) Buy the ux-US bundle (pair) and use one on ground, one on air?

I’ll tailor the final BOM line items to match your choice (including antennas/cables) so it’s purchase-order-ready.

Perfect — Option 3 (the RFD900ux-US telemetry bundle) is absolutely the best fit for your project. Below is your finalized, procurement-ready BOM, plus what’s included, what you still need (very little), and why this option satisfies your NDAA / Blue-UAS compliance requirements.

Everything here is backed by the search results.

# ✅ Final NDAA-Compliant Bill of Materials (Option 3: RFD900ux-US Bundle)

## 1. RFD900ux-US Telemetry Bundle (Pair of Modems + Cables + Antennas)

Product: RFD900ux-US Telemetry Bundle
Source information:

- Explicitly listed as DoD Blue UAS approved, compliant with FY20 NDAA Section 848, FY23 NDAA Section 817, and the American Security Drone Act, and validated as cyber-secure for government use.

- Specifically states “2024 NDAA compliant, all chips and components sourced and assembled in Allied nations.” [fcc.report]

### Included in the bundle:

The RFD900ux-US bundle includes everything you need for both the drone (air) and laptop (ground):

- 2× RFD900ux-US radio modems
(These are the compact, lightweight ux-series radios; fully compatible with the x-series but optimized for airborne use.)

- 6× total 900 MHz antennas:

  - 1× quarter-wave monopole (2.1 dBi)

  - 1× right-angle monopole (2.1 dBi)

  - 4× half-wave dipole antennas (3 dBi)

- FTDI USB Cable (for the ground station USB→UART connection)

- u.FL → RP-SMA antenna pigtails (for both modems)

- 900ux wiring harnesses, including:

  - 900ux → 8-way socket cable

  - Multi-cable for UART/power (150 mm) [fcc.report]

This bundle gives you:

- Air unit ✔

- Ground unit ✔

- All antennas ✔

- All pigtails ✔

- All wiring ✔

- USB ground adapter ✔

- NDAA compliance assurance ✔

- Blue-UAS approval ✔

This means your procurement process becomes extremely simple — you buy ONE bundle.

# ✅ 2. (Optional) Power Accessories

Your RFD900ux-US can draw up to ~800 mA peak at +30 dBm TX power (per ux datasheet).
Since your Raspberry Pi power budget is tight but your drone has huge payload capacity, you may optionally include: [fmshobby.com]

### 5V BEC Regulated Power Module (Recommended)

Use if:

- Your Pi’s 5 V rail cannot supply a comfortable additional ~1 A peak

- You want to isolate RF power transients

Not NDAA-restricted; choose any US-available brand (Castle Creations, Pololu D24V series, etc.)

# 📡 Why Option 3 Is the Right Call (Recap)

### 1. NDAA & Blue-UAS Compliance

The RFD900ux-US bundle explicitly states:

- “Compliant with FY20 NDAA Section 848, FY23 NDAA Section 817, American Security Drone Act"

- “2024 NDAA compliant, all chips/components sourced/assembled in allied nations” [fcc.report]

This is a much stronger compliance statement than the standard RFD900x-US listings.

### 2. Complete Kit — No Missing Parts

Unlike many RFD900x kits (which often do NOT include antennas or USB cables), the ux-US bundle includes: [worldronemarket.com]

- All antennas

- All pigtails

- All wiring harnesses

- Ground-side FTDI USB cable

### 3. Perfect for Drone Integration

The RFD900ux specs:

- Compact footprint: ~21–33 mm per side depending on carrier

- Very lightweight (~10 g)

- Supports air data rates up to 224–500 kbps depending on firmware version

- Long-range FHSS link: 40 km+ LoS documented in multiple sources [fmshobby.com], [unmannedtech.us]

### 4. Fully Compatible With Future Expansion

You can later mix in RFD900x full-size units or upgrade firmware for mesh networks; the ux and x series are interoperable in point-to-point mode. [Microsoft…- RFDesign]

# 🛠 Wiring Diagram — Summary from Included PDF

You now have a PDF in your ZIP, but here’s the quick text recap:

Raspberry Pi → RFD900ux-US (Air Unit)

- Pi GPIO14 (TXD, pin 8) → RFD RX

- Pi GPIO15 (RXD, pin 10) ← RFD TX

- Pi 5 V → RFD 5 V Input

- Pi GND → RFD GND

(Using the 900ux multi-cable included in the kit.) [fcc.report]

Antenna: use the u.FL → RP-SMA pigtails + provided dipoles, mounted vertically.

# 📦 Final BOM for Procurement (Copy/Paste Ready)

### Primary Hardware

| Qty | Item | Notes |
| --- | --- | --- |
| 1 | RFD900ux-US Telemetry Bundle | Includes 2× ux-US radios, 6 antennas, FTDI USB cable, wiring harnesses; NDAA + Blue-UAS compliant [fcc.report] |

### Optional but Recommended

| Qty | Item | Notes |
| --- | --- | --- |
| 1 | 5V 3A BEC regulator | Optional; ensures stable supply for the air-side radio |
| 1 | Spare u.FL → RP-SMA pigtails | Optional; helpful for field repairs |
| 1 | Tripod or mast for ground antenna | Optional; improves link reliability over vegetation or obstacles |

This whole system is extremely lightweight and will fit easily beneath your 5-lb remaining drone payload capacity.

# 🎉 You’re Fully Set

- You have the full ZIP package with working code & PDFs.

- You have a clean BOM meeting your NDAA/Blue-UAS constraints.

- You’ve chosen the best radio bundle for your flight envelope.

If you'd like, I can also prepare:

### ✔ A procurement-formatted PDF (with just BOM + direct links)

### ✔ A version of the strip-chart viewer that auto-detects COM ports

### ✔ A GPS-enabled methane plume mapping viewer (for future phases)

Just tell me what you'd like next!

Below is your clean, procurement-ready shopping list organized exactly as requested:
✔ Item
✔ Use in the project
✔ Link to purchase
✔ Required vs. Optional

All product info is sourced directly from the search results.

# 🛒 NDAA-Compliant Shopping List (Option 3: RFD900ux-US Bundle)

| Item | Use in This Project | Purchase Link | Required? |
| --- | --- | --- | --- |
| RFD900ux-US Telemetry Bundle (pair) | Provides air + ground radios, all required 915 MHz antennas, u.FL→RP-SMA pigtails, and FTDI USB cable for laptop connection. Certified NDAA compliant and Blue-UAS approved. | NW Blue – RFD900ux-US Telemetry Bundle | Required |
| 5 V 3A BEC Voltage Regulator (any NDAA-safe manufacturer, e.g., Castle Creations, Pololu) | Optional clean power source for RFD900ux on the drone if you do not want to power it directly from the Pi’s 5 V rail. Helps avoid brownouts during high-power RF bursts (~800 mA peak TX). | Generic—choose from US suppliers such as Castle Creations or Pololu (not in search results; no citation required). | Optional |
| Extra u.FL → RP-SMA Pigtails | Spare cable(s) in case of field damage—u.FL connectors are delicate. Bundle already includes some. | Included items listed here: NW Blue – RFD900ux-US Bundle | Optional |
| Tripod or Mast for Ground Antenna | Improves ground station link quality if operating over obstructions (trees, buildings, sagebrush, vehicles). | Can be sourced from any camera/ham radio supplier (no specific vendor in search results). | Optional |

# ✔ Why the Telemetry Bundle Is the Only Required Purchase

The RFD900ux-US Telemetry Bundle already includes:

- Two RFD900ux-US radios (air + ground)

- 6 total antennas (quarter-wave, right-angle, and dipole)

- FTDI USB cable for ground station

- u.FL → RP-SMA pigtails

- All wiring harnesses

  - 8-way socket cable

  - 150 mm multi-cable

- NDAA & Blue-UAS compliance certification [fcc.report]

This is why the bundle is the only “required” line item — it covers 100% of your drone-to-ground telemetry needs and satisfies all government compliance rules.

# If you want next:

I can generate:

### ✔ A formatted PDF “Procurement Sheet”

### ✔ A cart-ready version with quantities & notes for purchasing staff

### ✔ A version including estimated costs

Just tell me what you'd prefer!

Great — here is your procurement-ready sheet with direct purchase links, prices, uses, and required/optional status.
All prices and product details come from the verified search results.

# 📄 NDAA-Compliant Procurement Sheet

## 1. Required Items

| Item | Price | Use in Project | Purchase Link | Required? |
| --- | --- | --- | --- | --- |
| RFD900ux-US Telemetry Bundle (Pair) | $305.00 | This is the core communication system. Provides both air + ground radios, all antennas, u.FL→RP-SMA pigtails, all wiring harnesses, and a FTDI USB cable for ground-station connection. Fully NDAA-compliant and Blue-UAS approved (meets FY20 & FY23 NDAA sections and the American Security Drone Act). Enables live sensor streaming from drone → laptop. | RFD900ux-US Telemetry Bundle @ NW Blue [nwblue.com] | Required |

## 2. Optional Support Items

| Item | Price | Use in Project | Purchase Link | Required? |
| --- | --- | --- | --- | --- |
| Additional RFD900u Antenna Cables (RPSMA) | $5.99 | Spare antenna pigtails (helpful if an antenna cable is damaged in field use). Bundle already includes several, but extras are advisable. | https://irlock.com/products/rfd900u-antenna-cable-rpsma [irlock.com] | Optional |
| RFD900ux Multi-Cable (150 mm) | $8.99 | Extra wiring harness identical to the one in the bundle. Useful as a backup or for building a second airframe. | https://irlock.com/products/rfd900ux-multi-cable-150mm [irlock.com] | Optional |
| RFD 868ux/900ux to 8-Way Socket Cable | $10.00 | Additional wiring harness for integration flexibility (bundle includes one). | https://irlock.com/products/868ux-900ux-to-8-way-socket-cable-ux [irlock.com] | Optional |
| Ground Antenna Tripod / Mast | Varies ($20–$60) | Improves line-of-sight performance during testing or in uneven terrain. Not specific to RFD systems; any standard tripod works. | Choose any non-restricted tripod (not covered in search results). | Optional |
| 5V 3A BEC Regulator (Non-RFD Accessory) | ~$15–$30 | If you prefer to power the air radio separately from the Pi 5V rail, this provides clean, isolated power. Useful but not required. | Choose Pololu / Castle BEC from US suppliers. | Optional |

# ✔ Summary

### The only REQUIRED procurement item for your NDAA-compliant telemetry system:

### ✅ RFD900ux-US Telemetry Bundle — $305.00

Includes everything you need for drone ↔ ground digital data streaming.

### Everything else is optional convenience or redundancy.

If you'd like, I can also produce:

📄 A one-page PDF procurement sheet
📦 A bundle cart with quantities
🔎 A cost breakdown for budgeting
📝 A justification paragraph for grant or compliance documentation

Just tell me what you want next!

Yes — you can absolutely use the IR-LOCK RFD900ux-US Modem Bundle to visualize your Raspberry Pi SD-card data live while the drone is flying.
And this is fully supported by what we know about the bundle.

Here’s the breakdown, with citations.

# ✅ 1. The IR-LOCK RFD900ux-US Modem Bundle provides the same telemetry capability you need

The IR-LOCK listing explicitly states the kit includes:

- Two RFD900ux-US telemetry modems

- All required antennas

- FTDI USB ground-station cable

- All wiring harnesses for integration

These radios operate over 902–928 MHz and support long-range digital telemetry identical to the NW Blue bundle, because they use the same RFD900ux-US modems.

# ✅ 2. The radios offer a transparent digital serial link — perfect for streaming your methane sensor data

The RFD900ux-US modems are designed for:

- Long-range digital data communication

- TTL UART input/output

- FHSS (frequency hopping)

- High-reliability telemetry for UAVs [rcforces.com]

The Pi simply sends serial lines via UART → air radio → ground radio → USB into your laptop.
This is exactly the architecture you’re already set up for.

# ✅ 3. Real-time visualization is exactly what these radios are intended for

The RFD900ux-US supports:

- Air data rates up to 224 kbps

- Full serial pass-through from aircraft to ground

- FCC-approved long-range communication
citeturn16search41

Your methane data stream is extremely small — just timestamp + float value — so it fits easily within the modems’ capabilities.

You can absolutely:

- Continue logging to the SD card

- Simultaneously livestream the readings to your laptop

- Plot them in the scrolling strip-chart viewer included in your ZIP package

# ✅ 4. This bundle is “in stock,” making it a practical replacement

The IR-LOCK listing shows:

- “In stock (39 units), ready to ship”

This makes it the best available substitute for the NW Blue bundle (which is on pre-order).

# 🟢 Bottom line

Yes — the IR-LOCK RFD900ux-US Modem Bundle (FCC Approved) is fully compatible with your system and will allow you to:

### ► Stream your methane sensor data live

### ► Visualize it on your laptop in real time

### ► Continue logging to the Raspberry Pi’s SD card

### ► Maintain NDAA-friendly hardware

### ► Get hardware that ships now, not on backorder

It is functionally the same as the NW Blue version for your purposes.

If you’d like, I can also:

- Update your Procurement Sheet PDF using this IR-LOCK bundle

- Create a wiring diagram specifically labeled for the IR-LOCK harness

- Provide a step-by-step integration checklist

- Create a ground station auto-connect dashboard

Just tell me!

Great question — and it’s an important one because the wording on the IR-LOCK page can easily be misunderstood.

Here’s exactly what that line means, in plain English, based strictly on the IR-LOCK product listing.

# ✅ What the line means

The IR-LOCK product description states:

“It requires the RFD900ux PIXHAWK2 to RFD900 Telemetry Cable – 150/300mm to interface to your application.”

This note is NOT aimed at Raspberry Pi users.
It is specifically directed at Pixhawk flight controller users.

### In other words:

If you are trying to plug this radio into a Pixhawk 2.x flight controller, you must use a specific Pixhawk-to-RFD cable, which IR-LOCK sells separately.

### For your project (Raspberry Pi + custom methane payload):

➡️ You do NOT need that Pixhawk cable.

Because:

- The bundle already includes:

  - 900ux → 8-way socket cable

  - RFD900ux multi-cable (150 mm)

  - FTDI USB cable for ground

- The RFD900ux communicates over a simple UART interface

- Your Raspberry Pi already uses TX/RX/GND/5V pins for this

- You will wire the Pi directly to the RFD900ux using the included multi-cable

# 🧠 Why IR-LOCK includes that sentence

The RFD radios are widely used in the drone industry, and IR-LOCK is a major Pixhawk accessories supplier. Many of their customers:

- Fly with Pixhawk controllers

- Use MAVLink telemetry

- Need the correct 6-pin or 10-pin JST-GH cable for telemetry port TELEM1/TELEM2

Therefore, the product page reminds those customers:

“If you're plugging this into a Pixhawk, you need a Pixhawk-specific cable.”

That’s all it means.

# 🟢 Your setup does NOT require the Pixhawk cable

Because you are:

- Using a Raspberry Pi, not a Pixhawk

- Sending custom methane sensor data, not MAVLink

- Already connecting via UART

- Not using flight-controller serial ports

You will use:

### ✔ The RFD900ux multi-cable (150mm) for Pi → Air Unit

### ✔ The FTDI USB cable for Laptop → Ground Unit

Both are included in the IR-LOCK bundle.

So nothing extra is needed.

# 🛠 Wiring Summary for Your Raspberry Pi

You will connect:

| RFD900ux-US Pin | Raspberry Pi Pin |
| --- | --- |
| TX | GPIO 15 (RXD) |
| RX | GPIO 14 (TXD) |
| 5V | 5V pin (or BEC) |
| GND | Any Pi ground pin |

The included 900ux multi-cable provides these pins pre-terminated.
citeturn16search42

# 🟣 Bottom Line

### ✔ Yes — you can use the IR-LOCK RFD900ux-US Modem Bundle exactly as planned.

### ✔ You do not need the Pixhawk cable.

### ✔ That line only applies to Pixhawk users, not Raspberry Pi telemetry users.

If you’d like, I can:

- Make a wiring diagram specifically for the IR-LOCK cable labeling

- Confirm your Pi UART settings

- Update your procurement sheet using the IR-LOCK bundle

- Generate instructions for your field deployment kit

Just tell me!

Absolutely—here’s a start-to-finish implementation guide for streaming and visualizing methane telemetry from a Raspberry Pi 5 over a 915 MHz RFD900ux-US digital link (the IR-LOCK bundle), while simultaneously logging to the SD card. It includes: bill of materials, wiring, configuration, air- and ground-side code, testing, and field best practices.

Sources used throughout (cited inline):

- IR-LOCK RFD900ux-US Modem Bundle—contents and what’s included. [irlock.com]

- NW Blue listing (for cross-checking bundle contents wording and band/region). [nwblue.com]

- RFD900ux datasheet / manuals—UX electrical I/O (5 V supply, 3.3 V TTL UART), data rates, diversity. [RFDesign RFD900ux], [manualslib.com], [manualshelf.com]

- Raspberry Pi 5—UART on GPIO14/15, enabling serial/overlays background (GPIO 14=TXD, 15=RXD; serial enabling). [pinout.xyz], [raspberrytips.com], [forums.ras…errypi.com]

## 0) What you will build

Air (on the drone)\ Raspberry Pi 5 + your methane sensor → RFD900ux-US (air) via UART.\ Pi logs data to SD and streams live CSV lines over UART.

Ground\ RFD900ux-US (ground) → laptop (USB via FTDI in the bundle) → Python app shows a scrolling strip-chart and saves a mirrored CSV.

The IR-LOCK bundle already includes two RFD900ux-US radios, multiple 900 MHz antennas, the FTDI USB cable, and the ux wiring harnesses—so you’re not chasing parts. [irlock.com]

## 1) Bill of materials (NDAA-friendly)

- RFD900ux-US Modem Bundle (IR-LOCK) — includes:

  - 2× RFD900ux-US radios, 900 MHz antennas (quarter-wave, right-angle, dipoles), FTDI USB cable, 4× RFD900u antenna pigtails, 900ux→8-way socket cable, RFD900ux multi-cable (150 mm). [irlock.com]

- Raspberry Pi 5 (with Raspberry Pi OS and SD card). Pi 5 keeps the classic 40-pin header with UART on GPIO14/15 (TXD/RXD) as on earlier Pis. [pinout.xyz]

- (Optional) 5 V BEC if you prefer isolated power for the air radio (not required; see §8). (General rationale: UX radios can draw ~1 A peak at +30 dBm TX) [RFDesign RFD900ux]

## 2) Physical layout & wiring

### 2.1 Air unit wiring (Raspberry Pi 5 → RFD900ux-US)

RFD900ux-US signals (from UX datasheet / manuals): UART is 3.3 V TTL, power +5 V, common ground. [RFDesign RFD900ux], [manualslib.com]

Raspberry Pi 5 GPIO:

- GPIO14 (TXD) = Pin 8

- GPIO15 (RXD) = Pin 10

- 5 V = Pin 2 or 4

- GND = e.g., Pin 6\ (Standard UART mapping across Pi models; Pi 5 retains UART availability on 40-pin header.) [pinout.xyz]

Use the RFD900ux multi-cable (150 mm) and/or 8-way socket cable included in the IR-LOCK bundle to break out the UX pins. [irlock.com]

Connections (air):

Raspberry Pi 5 (40-pin)                RFD900ux-US (Air)

----------------------------------     ------------------------------

Pin 8   GPIO14 (TXD)   --------------> RX (3.3V TTL UART)

Pin 10  GPIO15 (RXD)   <-------------- TX (3.3V TTL UART)

Pin 2/4 5V             --------------> +5V

Pin 6   GND            --------------> GND

Antenna port(s)        <-------------- 900 MHz antennas (dipoles/monopoles included)

- Keep the antenna vertical and away from high-current wiring (ESCs). Diversity is supported via two antenna ports on UX. [RFDesign RFD900ux]

- If possible, mount the radio a few centimeters from the sensor head to minimize any RF coupling (UX runs at 915 MHz). (General best practice; UX power is up to +30 dBm.) [RFDesign RFD900ux]

### 2.2 Ground unit wiring (Laptop ↔ RFD900ux-US)

Use the included FTDI USB cable from the bundle: Laptop USB → FTDI → UX (ground unit). The UX on ground acts as a USB-serial device to your OS. [irlock.com]

Attach one of the included 900 MHz antennas to the ground radio; a small tripod mast is helpful but optional.

## 3) Configure the Raspberry Pi 5 for UART

On the Pi, you must enable the UART on GPIO14/15 and disable the login shell on serial so your code can use it.

Option A (raspi-config):

sudo raspi-config

# Interface Options → Serial Port:

#  - “Login shell over serial?” → No

#  - “Enable serial hardware?” → Yes

sudo reboot

(These steps are the standard approach on Pi OS to expose UART on /dev/serial0 or /dev/ttyAMA0.) [raspberrytips.com], [fleetstack.io]

Note on Pi 5 overlays: The Pi 5 introduces additional UART options and overlays (e.g., dtoverlay=uart0-pi5) to specifically map UART0 to GPIO14/15 as needed. If you later require this overlay, add to /boot/firmware/config.txt (Pi OS Bookworm) as:\ dtoverlay=uart0-pi5 then reboot. (Background discussed by Pi community; exact overlay usage may vary with OS image.) [forums.ras…errypi.com]

## 4) Set up the RFD900ux radios

Most UX radios ship FCC-locked in US region and will link with default parameters (pairing out of the box). (NW Blue & IR-LOCK note FCC-locked parameters for US variants; bundles ship pre-configured.) [nwblue.com], [irlock.com]

Quick sanity settings (recommended):

- Air (Pi) and Ground (laptop) radios: same NetID, same air data rate (e.g., 64 kbps to start), UART 57600 bps, TX power 20–100 mW for short-range tests (increase only if needed). (Parameter ranges per UX docs.) [RFDesign RFD900ux]

You can configure via AT commands or vendor tools later if needed; for basic CSV streaming, defaults typically work.

## 5) Air-side code (Pi): Log to SD and stream live over UART

The code below:

- Reads a methane value from read_methane() (stub—replace with your sensor call).

- Appends to /home/pi/methane_log.csv.

- Streams each reading as a CSV line timestamp,value over the serial link.

Uses the Pi’s UART device—commonly /dev/serial0 or /dev/ttyAMA0 depending on image/settings. (Pi UART enabling references) [raspberrytips.com], [fleetstack.io]

# airtx.py

#!/usr/bin/env python3

"""

Air-side transmitter for Raspberry Pi 5 (RFD900ux-US bundle)

- Logs methane data to SD card CSV

- Streams timestamped readings over UART to the RFD900ux air radio

"""

import time

import os

import csv

import serial

LOGPATH = os.environ.get('METHANELOG', '/home/pi/methanelog.csv')

SERIALPORT = os.environ.get('AIRSERIALPORT', '/dev/serial0')  # or /dev/ttyAMA0

BAUD = int(os.environ.get('AIRBAUD', '57600'))

PERIODS = float(os.environ.get('AIRPERIODS', '0.5'))  # 2 Hz default

# TODO: replace with your actual sensor read

def readmethane():

# e.g., return sensor.getppm()

return 1.234

def ensurelog():

new = not os.path.exists(LOGPATH)

f = open(LOGPATH, 'a', newline='')

w = csv.writer(f)

if new:

w.writerow(['timestamps', 'methanevalue'])

f.flush()

return f, w

def main():

ser = serial.Serial(SERIALPORT, BAUD, timeout=1)

f, w = ensurelog()

try:

while True:

ts = time.time()

val = float(readmethane())

# 1) Log locally

w.writerow([f"{ts:.3f}", f"{val:.6f}"])

f.flush()

# 2) Stream live

line = f"{ts:.3f},{val:.6f}\n"

ser.write(line.encode('utf-8'))

ser.flush()

time.sleep(PERIODS)

except KeyboardInterrupt:

pass

finally:

f.close()

ser.close()

if name == 'main':

main()

Tip: If you want it to auto-start, create a systemd service pointing to air_tx.py so it starts when the Pi boots. (General Pi practice; UART must be enabled as above.) [raspberrytips.com]

## 6) Ground-side code (laptop): Live scrolling strip-chart + mirrored CSV

The ground UX appears as a USB serial port via the included FTDI cable. (Bundle includes FTDI cable expressly for this.) [irlock.com]

# groundviewer.py

#!/usr/bin/env python3

"""

Ground-side viewer for RFD900ux-US link

- Opens USB serial from the ground radio

- Displays a live scrolling strip-chart (last N seconds)

- Mirrors received data into a local CSV

"""

import sys, csv, serial, collections

import matplotlib.pyplot as plt

import matplotlib.animation as animation

PORT = sys.argv[1] if len(sys.argv) > 1 else 'COM7'  # Windows example

BAUD = int(os.environ.get('GROUNDBAUD', '57600'))

MIRRORCSV = os.environ.get('GROUNDMIRROR', 'groundmethanelog.csv')

WINDOWSECONDS = int(os.environ.get('GROUNDWINDOWS', '300'))  # 5 minutes

def main():

ser = serial.Serial(PORT, BAUD, timeout=1)

mirror = open(MIRRORCSV, 'a', newline='')

mwriter = csv.writer(mirror)

if mirror.tell() == 0:

mwriter.writerow(['timestamps', 'methanevalue'])

mirror.flush()

data = collections.deque(maxlen=20000)

fig, ax = plt.subplots(figsize=(9, 4.5))

line, = ax.plot([], [], lw=2)

ax.settitle('Live Methane (strip-chart)')

ax.setxlabel('Time (s, relative)')

ax.setylabel('Methane (units)')

t0 = None

def update(frame):

nonlocal t0

# read all available lines

while ser.inwaiting:

raw = ser.readline().decode('utf-8', errors='ignore').strip()

if not raw:

continue

parts = raw.split(',')

if len(parts) != 2:

continue

try:

ts = float(parts[0]); val = float(parts[1])

except Exception:

continue

if t0 is None:

t0 = ts

data.append((ts - t0, val))

mwriter.writerow([f"{ts:.3f}", f"{val:.6f}"])

mirror.flush()

if not data:

return line,

# keep last N seconds

tmax = data[-1][0]

while data and (tmax - data[0][0]) > WINDOWSECONDS:

data.popleft()

xs = [p[0] for p in data]

ys = [p[1] for p in data]

line.setdata(xs, ys)

ax.setxlim(max(0, tmax - WINDOWSECONDS), tmax + 2)

ymin, ymax = min(ys), max(ys)

pad = max(1e-6, (ymax - ymin) * 0.1)

ax.setylim(ymin - pad, ymax + pad)

return line,

ani = animation.FuncAnimation(fig, update, interval=200)

plt.tight_layout()

plt.show()

mirror.close(); ser.close()

if name == 'main':

main()

Run examples:

- Windows: python ground_viewer.py COM7

- Linux: python ground_viewer.py /dev/ttyUSB0

- macOS: python ground_viewer.py /dev/tty.usbserial-XXXX

## 7) First-time bring-up checklist

- Bench test with both radios powered, antennas attached.

- On Pi: python3 air_tx.py (or start your service). Ensure /dev/serial0 is correct. (UART enabling as above.) [raspberrytips.com]

- On the laptop: check Device Manager (Windows) or ls /dev/ttyUSB* (Linux).

- Run the ground viewer with the correct port; confirm a live plot and that ground_methane_log.csv is growing.

- If no data: swap TX/RX lines on the Pi side (classic UART gotcha), confirm baud parity (57600, N, 8, 1), and ensure serial login shell is disabled. [raspberrytips.com]

## 8) Power, antennas, and radio parameters

- Power draw: RFD900ux peak TX currents up to ~1 A at +30 dBm; RX ~45–60 mA typical. Use Pi 5 V rail if it’s solid; otherwise consider a 5 V BEC to isolate the radio. [RFDesign RFD900ux]

- Antenna diversity: UX has two u.FL ports (bundle includes pigtails/antennas). Use matching dipoles; keep vertical and clear of power wiring. [RFDesign RFD900ux]

- US band: Ensure US-locked parameters (902–928 MHz ISM) are used, which is how ux-US bundles ship. [irlock.com], [nwblue.com]

- Initial TX power: Start low (e.g., 20–100 mW) for bench and short-range flight tests; scale only if needed. (UX supports up to +30 dBm.) [RFDesign RFD900ux]

## 9) What each part does (and how you’ll use it)

- RFD900ux-US (Air): Converts Pi’s UART stream into a long-range FHSS RF link at 915 MHz. You power it (5 V), wire Pi TX→UX RX and Pi RX←UX TX, and attach antennas. [RFDesign RFD900ux]

- RFD900ux-US (Ground): Receives the RF stream and presents a USB serial port to the laptop (via FTDI cable, included). Your plotting script reads that serial port. [irlock.com]

- Antennas & pigtails: The bundle includes monopoles/dipoles and u.FL→RP-SMA lines for both radios. Attach firmly and keep them vertical for best link. [irlock.com]

- Raspberry Pi 5: Runs your methane acquisition code, logs to methane_log.csv, and sends CSV lines over UART. The Pi’s GPIO14/15 pins carry the serial, once enabled. [pinout.xyz], [raspberrytips.com]

## 10) Troubleshooting quick wins

- No serial device on laptop: Ensure the FTDI cable is connected to the ground radio, drivers are present, and note the correct port (COMx / /dev/ttyUSB*). (FTDI cable is listed in the bundle contents.) [irlock.com]

- No live data but file logging works: Check UART:

  - Is the login shell disabled and serial hardware enabled? [raspberrytips.com]

  - Are TX/RX crossed properly?

  - Does Pi show the port (/dev/serial0) and can you open it?

- Intermittent drops: Raise ground antenna height; reduce air data rate; increase TX power cautiously; ensure antennas are vertical and connections are snug. (UX supports adjustable air data rates and TX power.) [RFDesign RFD900ux]

## 11) (Optional) Service file for auto-start on the Pi

# /etc/systemd/system/airtx.service

[Unit]

Description=Air-side methane telemetry (RFD900ux)

After=network.target

[Service]

Type=simple

User=pi

ExecStart=/usr/bin/python3 /home/pi/airtx.py

Restart=on-failure

RestartSec=3

[Install]

WantedBy=multi-user.target

Enable:

sudo systemctl enable --now air_tx.service

## 12) Field checklist (pre-flight)

- Radios on, antennas attached (never TX without antennas). UX supports two antennas—use both for diversity when possible. [RFDesign RFD900ux]

- Ground viewer running; confirm live scrolling trace for a minute.

- Log files: Confirm /home/pi/methane_log.csv is growing and ground mirror CSV is writing.

- Power sanity: If the drone has noisy power rails, consider the 5 V BEC for the air radio. [RFDesign RFD900ux]

## 13) Why this architecture works

- RFD900ux gives a long-range digital link in the 902–928 MHz ISM band using FHSS, designed for UAV telemetry. It supports 3.3 V TTL UART, 5 V supply, and adjustable data rates & TX power. [RFDesign RFD900ux]

- The IR-LOCK bundle is turnkey: two modems, antennas, FTDI USB, and all the UX cables you need—no extra purchases required to connect a Pi and a laptop. [irlock.com]

- The Raspberry Pi 5 still exposes UART on GPIO14/15, and Pi OS provides standard tools to enable it and free it from the login console. [pinout.xyz], [raspberrytips.com]

### Want this packaged as a downloadable kit?

I can generate a ZIP containing:

- The two Python scripts (air_tx.py, ground_viewer.py)

- A PDF quick-start tailored to IR-LOCK’s cable names

- A PDF wiring diagram with the exact pin labels (Pi5 ↔ RFD900ux-US)

- An optional systemd file

If you say the word, I’ll build it now and share the download link.

Awesome — I’ve built a downloadable kit for your Raspberry Pi 5 + RFD900ux-US setup.

### ⬇️ Download

- Pi5 + RFD900ux Telemetry Kit (ZIP) — code + PDFs + service file\ pi5_rfd900ux_kit.zip

What’s inside

- air_tx_pi5.py – Pi (air-side): logs to SD and streams CSV over UART

- ground_viewer.py – Laptop (ground-side): live scrolling strip-chart + mirrored CSV

- services/air_tx.service – optional systemd auto-start on the Pi

- QuickStart_Pi5_RFD900ux.pdf – 1-page setup guide

- WiringDiagram_Pi5_to_RFD900ux.pdf – Pi5 ↔ RFD900ux wiring

## Quick how-to (recap)

- Wire the air unit (using the IR-LOCK bundle’s 900ux cables):

  - Pi5 Pin 8 (GPIO14, TXD) → RFD900ux RX

  - Pi5 Pin 10 (GPIO15, RXD) ← RFD900ux TX

  - Pi5 5 V (Pin 2/4) → RFD900ux +5 V

  - Pi5 GND (Pin 6) → RFD900ux GND\ (RFD900ux uses 3.3 V TTL UART and 5 V supply. Antennas vertical; use included u.FL pigtails and 900 MHz antennas.) [us-prod.as…rosoft.com], [files.rfde…ign.com.au]

- Enable the Pi 5 UART:\ raspi-config → Interface Options → Serial → Login shell: No; Serial hardware: Yes → reboot\ (Pi 5 still exposes UART on the 40-pin header: GPIO14/15.) [shop.app], [rcnewb.com]

- Run the apps

  - Air (Pi): python3 air_tx_pi5.py

  - Ground (Laptop): python3 ground_viewer.py <PORT> (e.g., COM7 or /dev/ttyUSB0)\ (Ground unit uses the included FTDI USB cable from the bundle.) [files.rfde…ign.com.au]

- Optional auto-start on the Pi:\ Copy services/air_tx.service to /etc/systemd/system/ and enable it:

sudo cp services/airtx.service /etc/systemd/system/

sudo systemctl enable --now airtx.service

## Notes & best practices

- Start low TX power (20–100 mW) for bench/short-range; increase only as needed. (RFD900ux supports up to +30 dBm.) [us-prod.as…rosoft.com]

- Ensure serial console is disabled and serial hardware is enabled so /dev/serial0 is free for your app. [shop.app]

- The IR-LOCK bundle includes: 2× RFD900ux-US, multiple 900 MHz antennas, FTDI USB cable, 900ux→8-way socket cable, RFD900ux multi-cable, and pigtails—no extra parts required to connect Pi↔air radio and ground radio↔laptop. [files.rfde…ign.com.au]

If you want, I can also generate:

- A Pi-5-specific service installer script (install_service.sh)

- A GPS-enabled map viewer (methane vs. position) for your next phase

- A grant-ready quick-start PDF with NDAA/Blue-UAS language (sourced from vendor listings)

Just say the word and I’ll add them to the ZIP.